|  | ##### Fakultät für Physik ###### Isotopenforschung und Kernphysik |
| --- | --- |

E I N L A D U N G
zum

V E R A - S E M I N A R

von

#### Martin Martschini

Fakultät für Physik - Isotopenforschung, Universität Wien

AMS of 36Cl with the VERA 3 MV Tandem Accelerator

AMS of 36Cl (t1/2 = 0.30 Ma) at natural isotopic concentrations requires high particle energies for the separation from the stable isobar 36S. Up to now, these energies were usually provided by tandem accelerators with terminal voltages > 5 MV. At VERA we have recently achieved 36S suppression factors above 104 at 3 MV terminal voltage. We use terminal foil stripping and a detection setup consisting of a split-anode ionization chamber and a silicon strip detector. To further increase the 36S suppression we investigated energy loss straggling in various counting gases and the effect of “energy focusing” below the maximum of the Bragg curve. Comparison of experimental data with simulations and published data showed how energy loss, energy straggling and angular scattering determine the 36S suppression.

In addition, we improved ion source conditions, target backing materials, and the cathode design with respect to sulfur output and cross contamination. A thorough investigation of the memory effect of our ion source lead to a better understanding of our blank value. Currently it is 36Cl/Cl  510-16 if measured together with samples with a ratio of 10-12 or below, and 36Cl/Cl  310-15 when 10-11-samples are used in the same sample wheel. We achieve an injector to detector efficiency for 36Cl-ions of 8%. This comprises 16% stripping yield for the 7+ charge state in the accelerator and about 50% loss in the detection setup. We can now demonstrate that 36Cl measurements at 3 MV terminal voltage are competitive to measurements at larger tandems.

Recently we started to explore the potential of selective laser-optical filtering of sulfur and chlorine by photodetachment directly in the ion source. First experiments in Gothenburg yielded surprising results. The aim of this talk is to present the advances in AMS of 36Cl achieved over the last 2 years.

### Donnerstag, 14. April 2011, 16:30 Uhr

1090 Wien, Währinger Str. 17, "Kavalierstrakt",
1. Stock, Victor-Franz-Hess Hörsaal

R. Golser                             W. Kutschera			E.M. Wild