Resonant Laser-SNMS on actinides for spatially resolved ultra-trace analysis

Clemens Walther, Leibniz Universität Hannover, Institut für

Radioökologie und Strahlenschutz

Transport mechanisms of actinides in natural systems are of major

relevance to evaluate their distribution in geological formations at

contaminated areas and storage sites. The new resonant Laser-SNMS

system at the IRS Hannover was developed to cover these specifications

by combining the high element selectivity of resonance ionization with

the non-destructive analysis of a static TOF-SIMS with spatial

resolution down to 70 nm. First mass spectra of synthetical uranium,

plutonium technetium and strontium samples demonstrated the expected

suppression of interfering elements and molecules. In MOX fuel Pu-238

and U-238 were successfully discriminated. Due to the very low sample

consumption isotope selective images of environmental sample material

is performed almost non-invasively. Examples include natural minerals

from Afghanistan close to Kabul containing enhanced levels of natural

radioactivity, particles from the Chernobyl exclusion zone and from

the evacuated zone close to the Fukushima Daichii nuclear power plant.