Laser Ablation Interface coupled to AMS for online 14C analysis of carbonates

C. Yeman1*, C. Welte1, B. Hattendorf2, M. Christl1, J. Koch2, L. Wacker1, A.H. Andrews3, R. Witbaard4, D. Günther2, H.-A. Synal1

1. Laboratory of Ion Beam Physics, ETH Zurich, 8093 Zurich, Switzerland

2. Laboratory of Inorganic Chemistry, ETH Zurich, 8093 Zurich, Switzerland

3. NOAA Fisheries – Pacific Islands Fisheries Science Center, HI 96818, USA

4. NIOZ Royal Netherlands Inst. for Sea Research, 1797 SZ 't Horntje, The Netherlands

A novel method for direct and quasi continuous 14C analysis of carbonates was developed, where a laser ablation (LA) interface is coupled to the gas ion source of the MICADAS (MIniCArbonDAtingSystem) accelerator mass spectrometer (AMS) at the Laboratory of Ion Beam Physics, ETH Zurich. By focusing a pulsed laser beam (ArF excimer laser 193 nm, 200 – 250 Hz) on the sample’s surface, CO and CO2 is produced, which is directly and continuously introduced into the gas ion source of the AMS and analyzed online for radiocarbon. Scanning along the growth axis of a naturally grown carbonate sample such as stalagmites, corals, shells etc. a quasi-continuous record is obtained. The high flexibility of this novel approach allows to choose the integration time and thereby measurement precision and spatial resolution during offline data reduction. Compared to standard 14C sample preparation methods the LA-AMS setup has the advantages of high sample throughput, high spatial resolution, less material usage and minimal sample preparation.